## Project Requirements Document: Bluetooth Headphones

- Overview

  - Objectives: Why are we doing this project? What is the purpose?

The purpose of this project is to create a fully functional Bluetooth headphone system that demonstrates the integration of wireless audio transmission, user interface features, and sound quality. The objectives include:

Learning about Bluetooth communication protocols and implementation.

Gaining hands-on experience with embedded systems and enclosure design, particularly with the TM4C123GH6PM microcontroller and ESP32-DevKitC. Along with two of the TFT ST7735R LCD screens.

Developing skills in digital signal processing and audio amplification for improved sound output.

  - Interactions with Existing Systems: Include this if you are connecting to another board

  - Terminology: Define domain specific terms used in the document.

SPI - SPI is a synchronous serial communication protocol used for short-distance communication, primarily in embedded systems. It enables communication between a master device and one or more slave devices using four primary lines

I2C - I2C is a multi-master, multi-slave, synchronous serial communication protocol used for connecting low-speed devices in embedded systems. It requires only two wires

DAC - A DAC is an electronic component that converts digital signals (usually binary) into analog signals. It takes discrete values and outputs a continuous voltage or current that represents those values, enabling the representation of digital audio, video, and other signals in a form that can be understood by analog devices (like speakers or displays)

Bluetooth - Bluetooth is a wireless technology standard used for short-range communication between devices. It operates in the 2.4 GHz ISM band and allows for data exchange over distances typically up to 100 meters, depending on the Bluetooth class. Common applications include wireless audio streaming, file transfer, and connecting peripheral devices like keyboards, mice, and headphones

Voltage Regulator - A voltage regulator is an electrical device that maintains a constant output voltage level despite variations in input voltage or load conditions. It ensures that electronic components receive a stable voltage supply, which is crucial for their proper operation. Voltage regulators can be linear or switching types, with switching regulators being more efficient for higher power applications

Signal Amplitude - Signal amplitude refers to the maximum extent of a signal's variation from its rest position or average value. In the context of electrical signals, it is often measured in volts and indicates the strength or intensity of the signal. In audio applications, higher amplitude correlates to louder sounds, while in digital signals, amplitude variations can represent different binary states

Periodic Interrupts - Periodic interrupts are signals sent to a processor at regular intervals to prompt it to execute a specific piece of code or routine. This mechanism is commonly used in real-time operating systems (RTOS) and embedded systems to handle tasks that require consistent timing, such as sampling data from sensors or controlling motors. The processor can manage multiple tasks efficiently by prioritizing interrupts based on their timing

Equalization - Equalization (EQ) in Digital Signal Processing (DSP) is a technique used to adjust the balance of specific frequency components in an audio signal. It involves modifying the amplitude of different frequency bands to enhance, reduce, or modify the sound quality and tonal characteristics of audio playback

- Functional Description

  - Functionality: What will the system do precisely?

    - Bluetooth Audio Streaming (connectivity)

    - Dual Speaker Output

    - User Interface with two LCD screens

    - Volume Control

    - Equalization

  - Performance: Define the measures and describe how they will be determined.

Audio Quality: Use an oscilloscope to analyze the output waveform of the speakers for frequency response of the audible hearing range: 20 Hz to 20 kHz.

SNR:  Measure the output while playing a test tone at a known level and record the noise level to calculate the SNR. The target is to achieve an SNR of at least 80 dB for high-quality audio playback

Response Time: Measurements: Use a logic analyzer to measure the time from pressing a button to the corresponding action (volume change, playback toggle, or equalizer adjustment)

Equalization Effectiveness: Measurements: Evaluate the impact of the equalizer on audio output using frequency response measurements with an oscilloscope.

Determination: Compare audio signals before and after equalization to ensure intended frequency adjustments are achieved. The equalizer should demonstrate noticeable changes in specific frequency bands, allowing users to tailor the audio output to their preferences.

  - Usability: Describe the interfaces. Be quantitative if possible

TFT Display Interface: A graphical user interface showing changing audio levels depending on the volume of the sound.

Control Buttons: 4 buttons (2 for each speaker) for volume control and equalization functionalities.

Bluetooth Connectivity (I2C)

DAC interface (SPI): Two separate DACs will be connected to process the information from the ESP32 to a corresponding speaker.

- Deliverables

  - Roles and Responsibilities: Who will do what? What is the purpose?

  - Reports: Simply state the reports for Labs 7 8,  and 10 will be written

  - Outcomes: What are the deliverables? How do we know when it is done?

    - A fully functional prototype of Bluetooth headphones

    - Successful pairing and audio playback

    - Two working LCDs that provide real-time information

    - Comprehensive documentation to include: schematics, code, and a final project report